THEORY OF ELASTICITY

A PROJECT REPORT ON

Submitted by

GAURAV YADAV AND SHREYASH

2502103019

M.TECH

Mechanical Department

Indian Institute of Technology, Indore

India

November 2025

# 1.PROBLEM STATEMENT                        The objective of this assignment is to implement an incremental, integration-point–level stress update for a chosen deformation motion using both:

- Small deformation framework, and

- Finite deformation (large strain) framework, with an objective (Jaumann) stress rate.

At each increment, the only input is the incremental deformation gradient

ΔF.

The deformation gradient evolves as prescribed in the assignment:

Fn+1=ΔF Fn,F0=I.

The goal is to compute stresses and strains for each increment and generate a single comparison plot of equivalent stress vs equivalent strain for both small and large deformation formulations. This plot must demonstrate:

- linearity of the small-strain model,

- geometric nonlinearity in the large-strain model,

- stress rotation effects introduced by the objective stress rate.

A simple shear deformation is selected for this study.

# 2.Constitutive Relation (Transversely Isotropic Elasticity)

The material considered is a transversely isotropic carbon-fiber composite. The principal material direction (fibre direction) is aligned with the global x1-axis.

The elastic constants used are:

E1=150 GPa(fibre direction)

E2=10 GPa(transverse direction)

ν12=0.25

G12=5 GPa

G23=3.5 GPa

The compliance matrix S inVoigt form is assembled using these constants and inverted to obtain the stiffness matrix:

C=S-1.

This stiffness matrix remains constant throughout the simulation since the material is linearly elastic.

Rate-form constitutive equation

As required by the assignment:

σ∇=C:d,

where:

σ∇is an objective stress rate,

d is the rate of deformation tensor.

Two formulations follow:

2.1 Small Deformation Framework

Geometry does not update, so:

σ∇≈σ,d≈ε.

Thus the incremental stress update is:

σn+1=σn+C Δε.

2.2 Large Deformation Framework (Jaumann Rate)

The Jaumann rate is defined as:

σ∇=σ-wσ+σw,

where w is the spin tensor.

Thus the large strain update becomes:

σn+1=σn+Δt (σw-wσ)+C Δε.

This introduces stress rotation and geometric nonlinearity.

# Motion χ: Description of the Deformation

A simple shear deformation is imposed using the target deformation gradient:

F=130010001

The deformation is applied in N = 50 increments:

ΔF=F1/N.

At each increment, the deformation gradient is updated:

# Fn+1=ΔF Fn.

# 4.Small vs Large Deformation frameworks

Small Strain

- Linearized kinematics

- No rotation update

- Stress grows linearly with strain

- Valid only for very small deformations

Large Strain (Jaumann Rate)

- Accounts for geometric nonlinearity

- Includes rotation of stress components

- Captures shear-induced rotations even for linear elastic materials

- Stress–strain response becomes nonlinear even though the material law is linear

This comparison is essential for the assignment, and both behaviours clearly appear in the final results.

# 5.Equilibrium Check

The motion used in this assignment is homogeneous, and the stress update at every increment is performed at a single integration point.
Since there are no spatial gradients in the motion:

∇⋅σ=0

holds automatically.

Therefore:

- The updated Cauchy stress is in equilibrium in the current configuration.

- The configuration used for equilibrium verification is the current configuration, because stresses are calculated directly in spatial coordinates and the Jaumann rate ensures proper rotational consistency.

# 6. Results, Plots, and Interpretation